RESEARCH ARTICLE

Fuzzy Lagrangian Proximal Policy Optimization for Reliable UAV Selection in Dynamic UAV-IoV Systems

Shubh Om · Pathireddi Vara Kalyan · K. Jairam Naik

# Abstract

The growth of the Internet of Vehicles (IoV) has increased the demand for reliable and low-latency communication in dynamic vehicular environments. Although unmanned aerial vehicles (UAVs) improve network coverage and connectivity, selecting the most suitable UAV remains a challenging task because of dynamic vehicle mobility, varying link quality, communication delay, and limited UAV energy. Existing UAV selection approaches mainly rely on fixed decision strategies or existing reinforcement learning methods, which often fail to satisfy multiple Quality of Service (QoS) requirements under changing network conditions. To overcome these limitations, this paper proposes a Fuzzy Lagrangian Proximal Policy Optimization (Fuzzy LP-PPO) framework for intelligent UAV selection in UAV-assisted IoV networks. The proposed methodology first performs hard feasibility screening to remove unsuitable UAV candidates based on communication constraints. The feasible candidates are then evaluated using a fuzzy priority mechanism that jointly considers signal strength, communication delay, packet delivery ratio (PDR), and residual energy. Finally, an adaptive Lagrangian optimization strategy updates constraint penalties dynamically, while the PPO agent learns an optimal UAV selection policy from the ranked candidate actions. The framework is implemented in Python and evaluated under representative UAV-IoV communication scenarios. Compared with a reward-aware heuristic under a common evaluation, the trained policy achieves a 48.1% increase in episode reward, a 7.8% reduction in average delay, a 0.39 percentage-point increase in PDR, and a 14.8% improvement in signal quality relative to existing methods. These results indicate  that the proposed Fuzzy LP-PPO framework provides an effective and adaptive solution for reliable, constraint-aware UAV-assisted IoV communication.

# Keywords

Internet of Vehicles · Unmanned aerial vehicles · Fuzzy logic · Lagrangian optimization · Proximal policy optimization · Reliable communication

# Introduction

The Internet of Vehicles (IoV) is an important part of intelligent transportation systems that enables communication among vehicles, roadside infrastructure, cloud servers, and traffic management services. By exchanging real-time information, IoV supports applications such as traffic monitoring, collision avoidance, route planning, emergency response, and autonomous driving (Kaiwartya et al., 2016). These applications require reliable communication with low delay because even a small interruption may affect the performance of safety-critical services. As the number of connected vehicles continues to increase, maintaining stable communication in highly dynamic traffic environments has become a major research challenge.

Unmanned Aerial Vehicles (UAVs) have recently been introduced to improve IoV communication by providing flexible wireless coverage in areas where terrestrial infrastructure is unavailable or overloaded (Shakhatreh et al., 2019). Compared with fixed roadside units, UAVs can quickly change their positions, extend communication coverage, and support temporary or emergency situations. These advantages make UAV-assisted IoV a promising solution for future intelligent transportation systems. However, UAVs operate with limited battery capacity, finite communication range, and continuously changing network conditions, making reliable communication management a challenging task (Cabreira et al., 2019).

One of the most important challenges in UAV-assisted IoV is selecting the most suitable UAV for each communication request. The selection process depends on several factors, including communication distance, signal strength, packet delivery ratio (PDR), communication delay, and the remaining energy of each UAV. These parameters often conflict with one another. For example, the nearest UAV may provide a strong communication link but have low residual energy, whereas another UAV may have sufficient energy but introduce higher communication delay. As vehicle mobility changes continuously, the communication quality also changes rapidly, making UAV selection a dynamic optimization problem rather than a simple decision based on a single parameter.

Several existing approaches have been proposed to address UAV selection. Traditional methods mainly use fixed strategies such as random selection, nearest-UAV selection, strongest-signal selection, or energy-aware greedy selection (Sun et al., 2023; Li et al., 2023). Although these methods are simple to implement, they usually optimize only one communication parameter and cannot adapt effectively to changing traffic conditions. Recent studies have applied Reinforcement Learning (RL) to improve decision making by allowing an agent to learn an optimal policy through interaction with the environment (Lowe et al., 2017). Among RL algorithms, Proximal Policy Optimization (PPO) has gained considerable attention because of its stable learning process and efficient policy updates (Schulman et al., 2017). However, existing RL-based methods still face difficulties in simultaneously satisfying multiple Quality of Service (QoS) requirements while adapting to different traffic scenarios.

Figure 1 : Illustrative operating scenarios used to evaluate Fuzzy LP-PPO: urban canyon, suburban crossroads, and emergency response.

To overcome these limitations, this paper proposes a Fuzzy Lagrangian Proximal Policy Optimization (Fuzzy LP-PPO) framework for intelligent UAV selection in dynamic UAV-assisted IoV networks. Instead of selecting a UAV using a single communication metric, the proposed framework jointly considers signal strength, communication delay, packet delivery ratio, and residual UAV energy during the decision-making process. A hard feasibility screening stage removes unsuitable UAV candidates based on predefined communication constraints. The remaining candidates are prioritized using a fuzzy inference mechanism that evaluates their communication quality. An adaptive Lagrangian optimization strategy continuously updates constraint penalties according to the current network conditions, while a PPO-based learning agent identifies the most suitable UAV from the ranked candidates. By combining these components, the proposed framework aims to improve communication reliability while maintaining QoS requirements under dynamic vehicular environments.

The proposed framework is evaluated using three representative UAV-IoV scenarios, namely Urban Canyon, Suburban Crossroads, and Emergency Response, which represent different traffic densities and communication conditions. The performance of the proposed method is compared with several existing UAV selection strategies using common evaluation metrics, including episode reward, communication delay, packet delivery ratio, and signal quality. The experimental results demonstrate that the proposed framework achieves improved communication performance while effectively handling dynamic network conditions and multiple QoS constraints.

The main contributions of this work are summarized as follows:

A Fuzzy Lagrangian Proximal Policy Optimization (Fuzzy LP-PPO) framework is proposed for intelligent UAV selection in dynamic UAV-assisted IoV networks. The framework combines hard feasibility screening, fuzzy priority evaluation, adaptive Lagrangian optimization, and PPO-based learning to jointly consider communication delay, packet delivery ratio (PDR), signal strength, communication distance, and residual UAV energy during UAV selection.

A scenario-aware UAV-IoV communication framework is developed to represent diverse traffic conditions. The proposed framework models Urban Canyon, Suburban Crossroads, and Emergency Response scenarios, enabling adaptive UAV selection under different traffic densities, mobility patterns, and Quality of Service (QoS) requirements.

The effectiveness of the proposed framework is validated through comprehensive performance evaluation. The proposed method is compared with conventional UAV selection strategies using common QoS metrics, including cumulative reward, communication delay, packet delivery ratio, and signal quality. The experimental results demonstrate that the proposed Fuzzy LP-PPO framework consistently improves communication reliability while maintaining efficient resource utilization in dynamic UAV-IoV environments.

The remainder of this paper is organized as follows. Section "Literature survey" reviews existing work on UAV-assisted IoV communication, intelligent UAV selection, fuzzy decision systems, and constrained reinforcement learning, together with the resulting research gaps and also fuzzy-priority estimation, adaptive Lagrangian reward formulation, and PPO-based policy optimization that together constitute the proposed Fuzzy LP-PPO framework.

The abbreviations used throughout this paper are listed in Table 1, while the principal variables employed in the system model and proposed Fuzzy LP-PPO formulation are summarized in the same table:

Table 1 List of symbols and abbreviations

| Symbol | Description |
| --- | --- |
| t | Current communication decision step |
| H | Number of interaction steps in one episode |
| N_ep | Number of training episodes |
| V(t) | Set of active vehicles at decision step t; one representative active vehicle is served per selection decision |
| U | Set of eight candidate UAVs |
| p_v(t) | Position of vehicle v at decision step t |
| p_u(t) | Three-dimensional position of UAV u at decision step t |
| d_v,u(t) | Euclidean distance between vehicle v and UAV u |
| R_comm | Maximum UAV communication range |
| S_v,u(t) | Signal-quality value of the vehicle–UAV link |
| D_v,u(t) | Communication delay of the vehicle–UAV link |
| P_v,u(t) | Packet delivery ratio of the vehicle–UAV link |
| E_u(t) | Residual energy of UAV u |
| s_t | State observed by the PPO agent at step t |
| a_t | UAV-selection action chosen at step t |
| r_t | Immediate reward obtained after action a_t |
| π_θ(a_t ¦ s_t) | PPO policy parameterized by θ |
| V_φ(s_t) | State-value function parameterized by φ |
| F_v,u(t) | Fuzzy priority assigned to candidate link (v,u) |
| g_i(t) | Violation magnitude of QoS constraint i |
| λ_i(t) | Adaptive Lagrangian multiplier for constraint i |
| η_λ | Learning rate used for Lagrangian multiplier updates |
| γ | Reward discount factor |
| ε | PPO clipping parameter |
| A_t | Generalized advantage estimate at step t |
| λ_GAE | Generalized advantage-estimation parameter |

# Literature survey

We begin by reviewing existing research on UAV-assisted Internet of Vehicles (IoV), Mobile Edge Computing (MEC), and Artificial Intelligence (AI)-based optimization techniques. The reviewed studies primarily focus on computation offloading, task scheduling, resource allocation, UAV trajectory optimization, and communication reliability using Deep Reinforcement Learning (DRL), Proximal Policy Optimization (PPO), Multi-Agent Reinforcement Learning (MARL), Graph Reinforcement Learning (GRL), and other intelligent optimization methods. The analysis of these studies helps identify the strengths, limitations, and research gaps in existing methodologies, providing the motivation for the proposed Fuzzy Logic-Based Lagrangian Proximal Policy Optimization (FLP-PPO) framework.

Chen et al. (2026) proposed a traffic-aware asynchronous trajectory-planning framework combining spatial-temporal graph attention and GNN-enhanced PPO for UAV scheduling and data delivery. The method was evaluated in a simulated UAV-assisted wireless network using generated heterogeneous traffic and topology data. It improved throughput, scheduling efficiency and convergence over conventional DRL approaches. However, it does not integrate fuzzy QoS evaluation with adaptive Lagrangian constraints for reliable UAV selection.

Wang et al. (2026) proposed LC-MAPO, which integrates Lagrangian constraint handling with a multi-agent TD3 actor–critic framework for secure task offloading in drone-assisted IoV. It was tested in a simulated multi-drone vehicular network using generated mobility, communication task and battery data. The method improved offloading efficiency while reducing collision and battery-constraint violations against unconstrained learning methods. However, its main objective is task offloading rather than fuzzy communication-aware UAV selection.

Alagha et al. (2025) proposed a block-chain and reinforcement-learning-enabled framework for UAV relay selection and coordination in the Internet of Vehicles. The method was evaluated using simulated vehicle–UAV interactions, Quality-of-UAV and Quality-of-Vehicle information, with PPO used for decentralised UAV coordination. It achieved more stable relay selection and better vehicle coverage and connectivity than benchmark methods. However, it does not jointly optimise signal reliability, delay, PDR and UAV energy through fuzzy Lagrangian learning.

Goudarzi et al. (2025) proposed an SDN-based UAV-assisted vehicular edge-computing framework using Soft Actor–Critic for trajectory, user association and task-offloading decisions. It was evaluated in simulated dense and infrastructure-limited vehicular environments using generated network and task data. The method achieved up to 87.2% energy savings in energy-limited scenarios and a 50% reduction in time-sensitive scenarios. However, it focuses on AoI and offloading costs rather than fuzzy reliability-based UAV selection.

Zakaryia et al. (2025) proposed MADDPG-DTLCM for task offloading and resource allocation in multi-UAV-assisted edge computing. The framework was tested through simulations using generated UAV positions, user tasks, computational capacities and communication conditions. It achieved an 18% higher task-completion rate and 12% lower latency than MADQN, conventional MADDPG and greedy offloading. However, UAV selection depends mainly on distance and computational capability rather than fuzzy communication-reliability indicators.

Wu et al. (2025) proposed a MAPPO framework for jointly optimising service caching, content caching, computation offloading and UAV trajectories in UAV-assisted vehicular networks. It was evaluated using a simulated multi-UAV IoV environment with generated vehicle requests, channel states and task information. The method achieved an approximately 82% cache-hit rate and higher cumulative reward than representative baseline strategies. However, it does not incorporate fuzzy priority assessment or Lagrangian-based reliability constraints.

Xu et al. (2025) proposed MATD3-TORA for joint task offloading, UAV movement, CPU-frequency control and resource allocation in multi-UAV MEC systems. The method was tested in a simulated network of mobile devices and UAVs using generated mobility, channel and computational-task data. It achieved lower processing latency and UAV energy consumption than MADDPG and edge-only schemes while showing stable convergence. However, it does not perform fuzzy communication-quality-based UAV selection.

Su and Liu (2025) proposed an improved DDPG-based partial-offloading framework with task-priority modelling for cloud–edge collaborative IoV. The method was evaluated in a simulated vehicular environment using generated task, location, storage and computing-resource data. It reduced latency by 59.46% and 67.39% compared with DQN and DDPG, while reducing energy consumption by 18.37% and 11.76%, respectively. However, it does not include UAV coordination or communication-aware UAV selection.

Mustafa et al. (2025) proposed SQ-DDTO, combining Dueling DDQN, prioritised experience replay and Sequential Quadratic Programming for task offloading and resource allocation in vehicular edge networks. It was evaluated through simulations considering vehicle speed, location, channel quality and task characteristics. The method reduced system cost by 25.1% over DQN and 16.67% over DDQN and DDPG, while reducing task loss by up to 37.06%. However, it does not consider UAV-assisted selection or fuzzy constraint management.

Xue et al. (2025) proposed PORA-MATD3 for partial task offloading, resource allocation and task migration in vehicular edge-computing networks. The method was evaluated through simulations using generated vehicle mobility, RSU, communication-distance and computational-task data. It reduced long-term delay and energy overhead compared with conventional multi-agent and local-processing schemes. However, it focuses on vehicular resource allocation and does not evaluate UAV reliability using fuzzy signal, PDR and energy parameters.

L. Wang et al. (2025) proposed a Vehicle Deep Q-Network combined with Vehicle Adaptive Feedback for priority-aware task offloading in vehicular edge computing. The framework was tested through simulations considering vehicle position, speed, computation resources, connection interruption and maximum-delay constraints. It achieved lower cost consumption and higher user-service satisfaction than traditional offloading algorithms. However, it uses discrete DQN decisions and does not support PPO-based multi-UAV reliability optimisation.

Zhang et al. (2025) proposed a cloud–edge–device framework integrated with multi-agent deep reinforcement learning for dynamic task offloading and resource allocation in vehicular networks. It was evaluated in PyTorch using a simulated environment with one cloud server, four MEC servers and twenty vehicles per MEC server. The method reduced energy consumption by 11%, increased task completion by 23%, and reduced delay by 50% compared with local and static offloading. However, no UAV reliability or fuzzy constraint mechanism was considered.

Bo and Zhao (2025) proposed a multi-agent deep-reinforcement-learning task-offloading strategy for vehicular edge computing with queuing and NOMA-based communication modelling. The method was evaluated through a simulated IoV environment using generated vehicle tasks, queue states, channel conditions and edge-resource data. It improved offloading efficiency, resource utilisation and delay performance over conventional strategies. However, it does not address UAV coordination or fuzzy link-reliability evaluation.

Li et al. (2026) proposed a three-stage framework combining ST-FGCN demand prediction, collaborative caching and Genetic A3C task offloading for vehicular edge computing. It was evaluated through simulations using generated spatial-temporal task demands and  edge-cache states . The method reduced long-term system cost and improved latency and energy efficiency over baseline caching and reinforcement-learning methods.

Deng et al. (2025) proposed FDRLGT, combining federated deep reinforcement learning with game theory for privacy-preserving task offloading in vehicular edge computing. The framework was tested through simulations with multiple vehicles, distributed local data, dynamic channels and edge servers. It reduced total system delay while protecting raw user data and avoiding homogeneous offloading decisions compared with baseline algorithms. However, it does not model UAV mobility, fuzzy communication reliability or adaptive safety constraints.

Pantaleon et al. (2025) proposed BRAVE and BRAVE-MARL for benefit-aware data offloading and UAV path planning in public-safety edge-computing environments. The framework was evaluated using simulated disaster areas containing multiple areas of interest, users and energy-constrained UAVs. It improved data collection, user QoS support and multi-UAV coordination compared with non-collaborative path-planning approaches. However, it targets disaster-response networks rather than high-mobility IoV and does not use fuzzy PPO-based selection.

Cui et al. (2025) proposed a two-phase cooperative offloading and migration framework using COMA-based multi-agent reinforcement learning for VEC and V2V environments. It was evaluated through simulations using generated vehicle mobility, link quality, RSU resources and computation-task data. The framework reduced latency by up to 54% and increased the task-completion rate by up to 15% compared with related schemes. However, the framework does not involve UAV-assisted communication or fuzzy reliability scoring.

Ye et al. (2025) proposed an opportunistic data-transmission approach for integrated UAV–intelligent connected vehicle networks containing selfish relay nodes. The approach was evaluated in simulated UAV–vehicle communications using generated mobility and forwarding-behaviour data. It improved transmission reliability and cooperation compared with conventional opportunistic methods. However, it does not provide reinforcement-learning-based UAV selection or fuzzy Lagrangian optimisation.

Zhang et al. (2026) proposed a Multi-Objective Decomposition Evolutionary Deep Reinforcement Learning (MODE-DRL) framework for computation offloading in UAV-assisted Mobile Edge Computing. The framework was evaluated in a simulated Internet of Vehicles environment using generated vehicular tasks and network data. It reduced task delay and energy consumption while improving task-completion performance compared with conventional DRL methods. However, it does not consider fuzzy communication reliability or adaptive Lagrangian-based UAV selection.

Yan et al. (2025) proposed a Deep Graph Reinforcement Learning (DGRL) framework for computation offloading in UAV-assisted Vehicular Edge Computing Networks. The method was evaluated in a simulated vehicular edge computing environment using generated mobility and computation-task data. It improved computation offloading efficiency and resource utilization compared with conventional DRL approaches. However, it does not incorporate fuzzy QoS evaluation or reliable UAV selection using Lagrangian optimization.

The reviewed tells the effectiveness of Deep Reinforcement Learning (DRL), Multi-Agent Reinforcement Learning (MARL), PPO, Graph Reinforcement Learning (GRL), and DDPG in improving UAV-assisted Internet of Vehicles (IoV) systems. Most researchers focused on computation offloading, task scheduling, UAV trajectory optimization, resource allocation, and energy-efficient communication. These approaches significantly reduced latency and energy consumption while enhancing network throughput, resource utilization, and Quality of Service (QoS). However, most existing methods primarily optimize performance without considering communication uncertainty and reliable UAV selection. In addition, fuzzy logic-based decision making and adaptive Lagrangian constraint optimization are rarely integrated into current frameworks. Therefore, there is a need for an intelligent framework that combines fuzzy logic with reinforcement learning to achieve reliable UAV selection, efficient task offloading, and improved communication performance in dynamic UAV-assisted IoV environments.

Table 2 Detailed review of 15 verified representative papers

| Paper | Methodology | Dataset / Environment | Result | Merits | Limitations |
| --- | --- | --- | --- | --- | --- |
| Uddin et al. 2025 | Survey of DRL-based task offloading in vehicular edge computing. | Vehicular edge computing, incl. EC, nearby vehicles, and UAV-assisted settings. | DRL reported as more adaptive than static offloading for dynamic ITS workloads. | Recent taxonomy and reward-design guidance. | Survey only; no new UAV-selection algorithm. |
| Li et al. 2025 | Hierarchical SAC-based partial offloading with high- and low-altitude UAV layers. | Simulated UAV-assisted vehicular edge computing. | Outperformed baselines in task completion, efficiency, and convergence. | Handles heterogeneous UAV resources and dynamic conditions. | Optimizes offloading/trajectory, not direct fuzzy UAV association. |
| Alagha et al. 2025 | Two-sided UAV relay selection with decentralized PPO coordination and blockchain traceability. | UAV-assisted IoV relay-selection simulation. | Improved relay stability, coverage, and connectivity over baselines. | Combines relay selection, coordination, and trust. | Blockchain and multi-UAV coordination add overhead. |
| Sun et al. 2024 | MINLP decomposition using splitting, KKT, and SCA for offloading/resource/trajectory control. | Multi-UAV-assisted MEC simulation. | Superior delay, energy, and offloaded-task performance vs benchmarks. | Strong optimization baseline with multiple objectives. | Complex solver pipeline; less adaptive online than RL. |
| Kim et al. 2024 | Cooperative MADRL with graph-attention actors for UAV-aided MEC. | UAV-aided MEC with ground IoT devices. | Reported better MEC performance than conventional methods. | Scalable message/solution action design. | Heavier graph-attention MARL than the proposed centralized selector. |
| Sun et al. 2023 | Knowledge-driven MARL with graph neural networks for IoV computation offloading. | Cybertwin-enabled IoV with RSU offloading. | Reported higher reward and scalability than competing methods. | Uses topology knowledge to improve interpretability. | RSU/cybertwin focus; no UAV energy-aware fuzzy priority. |
| Li et al. 2023 | FlexEdge digital-twin PPO for UAV-aided vehicular edge computing. | Digital-twin-driven UAV-aided VEC simulation. | Quick convergence and reduced system energy consumption. | Uses PPO for trajectory and resource adaptation. | Single UAV-server focus and not constraint-price based. |
| Tang and Wong 2020 | Distributed DQN/LSTM task offloading for MEC. | 50 mobile devices and five edge nodes. | Reduced dropped tasks by 86.4–95.4% and delay by 18.0–30.1% vs baselines. | Clear quantitative DRL offloading baseline. | General MEC, not UAV-IoV selection. |
| Schulman et al. 2017 | Clipped-surrogate PPO policy optimization. | MuJoCo and Atari RL benchmarks. | Stable policy updates with strong benchmark performance. | Simple, stable actor–critic foundation. | No built-in QoS constraint handling. |
| Lowe et al. 2017 | MADDPG centralized training with decentralized execution. | Cooperative and competitive multi-agent particle environments. | Improved multi-agent learning stability over independent learners. | Useful for multi-UAV coordination. | Continuous multi-agent control more complex than single UAV selection. |
| Yu et al. 2022 | MAPPO adaptation of PPO to cooperative multi-agent tasks. | StarCraft Multi-Agent Challenge and other cooperative tasks. | Demonstrated strong cooperative MARL performance. | Stable PPO-style multi-agent baseline. | Requires multi-agent training machinery not needed here. |
| Achiam et al. 2017 | Constrained Policy Optimization for safe RL. | Constrained control benchmarks. | Maintained constraint satisfaction while optimizing reward. | Important safe-RL foundation. | Trust-region method harder to implement than PPO with multipliers. |
| Stooke et al. 2020 | Responsive safety in policy optimization using Lagrangian methods. | Safe RL benchmark environments. | Improved response to safety violations during training. | Supports adaptive constraint prices. | Generic safe RL, not UAV-IoV specific. |
| Chen et al. 2016 | Game-theoretic distributed multi-user computation offloading for mobile-edge cloud computing. | Multi-user mobile-edge cloud computing simulation. | Demonstrated efficient distributed offloading with low coordination overhead. | Relevant optimization baseline for computation offloading. | Not a deep-RL or direct UAV-selection method. |
| Wang et al. 2022 | Delay-aware coordination of dependent microservices in mobile edge computing. | Mobile edge-computing service-coordination environment. | Reduced service delay through dependency-aware microservice coordination. | Relevant delay-aware edge-service baseline. | Not a multi-agent RL or UAV-IoV resource-allocation method. |

Table 3 Comparison of related UAV-IoV decision and learning methods

| Method or family | Decision mechanism | Main strength | Principal limitation for this project |
| --- | --- | --- | --- |
| Nearest-UAV heuristic | Minimizes instantaneous vehicle–UAV distance | Simple and often reduces path loss | Ignores PDR, residual energy, and long-term fleet balance |
| Strongest-signal heuristic | Maximizes instantaneous signal quality | Improves immediate link strength | Can overuse individual UAVs and neglect energy and delay |
| Energy-aware greedy | Prioritizes residual UAV energy | Protects low-energy UAVs | May select distant links with weak communication quality |
| Reward-aware greedy | Maximizes a fixed weighted QoS score | Considers several metrics with low complexity | Myopic and sensitive to manually fixed weights |
| DQN and variants | Learns action values for discrete UAV choices | Supports nonlinear and adaptive selection | No inherent constraint enforcement; scalability issues for joint actions |
| DDPG/TD3 | Actor–critic learning for continuous control | Suitable for trajectory, power, and resource decisions | Sensitive to exploration and reward design |
| MADDPG | Centralized critics with decentralized continuous actors | Supports cooperative multi-UAV control | Unconstrained policies may violate safety or QoS requirements |
| MAPPO/HAPPO | PPO-based cooperative multi-agent policy learning | Stable multi-agent updates and coordination | Greater architectural and training complexity than required for centralized UAV selection |
| CPO/RCPO | Constrained or Lagrangian policy optimization | Explicit treatment of safety costs | Does not provide fuzzy handling of gradual link-quality uncertainty |
| Clipped fuzzy-risk selection | Weighted clipped linear risk memberships | Interpretable and computationally efficient | Fixed mappings do not learn long-term policies or adapt constraint prices |
| Proposed Fuzzy LP-PPO | Weighted clipped risk-priority aggregation, adaptive Lagrangian penalties, and PPO learning | Stable, constraint-aware selection using delay, PDR, signal quality, distance, and residual energy | Current implementation assumes synthetic link models and centralized UAV selection |

The main research gaps identified from the existing literature are listed as follows:

- Most existing methods focus mainly on latency reduction and energy optimization, while communication reliability is often overlooked.

- Current reinforcement learning approaches do not effectively handle uncertainty in dynamic UAV-assisted IoV environments.

- Existing frameworks lack fuzzy logic-based decision making for intelligent UAV selection under varying network conditions.

- Many studies optimize only a single objective or limited objectives, resulting in poor trade-offs among latency, energy, reliability, and QoS.

- Adaptive Lagrangian constraint optimization is rarely integrated to satisfy multiple system constraints simultaneously.

- Most approaches assume ideal or static network conditions, limiting their applicability in highly dynamic vehicular scenarios.

- Existing methods provide limited support for reliable and adaptive UAV selection during frequent topology and traffic changes.

- There is a lack of an integrated framework that combines Fuzzy Logic, PPO, and Lagrangian optimization for robust task offloading and resource allocation.

From the literature review, it is observed that several researchers have proposed different techniques to improve computation offloading, resource allocation, and communication performance in UAV-assisted IoV systems. However, existing approaches still face challenges in reliable UAV selection, handling dynamic network conditions, and satisfying multiple optimization objectives. These limitations motivate the need for a more effective and intelligent solution. Therefore, the proposed Fuzzy Logic-Based Lagrangian Proximal Policy Optimization (FLP-PPO) framework is developed to address these challenges. The next section presents the proposed methodology, including its architecture, workflow, mathematical model, and algorithm in detail.

# Our Method

In this section, we proposed a Fuzzy Lagrangian Proximal Policy Optimization (FL-PPO) methodology which is designed to achieve reliable UAV selection in dynamic UAV-IoV systems. It combines Fuzzy Logic to evaluate UAV reliability under uncertain network conditions with Proximal Policy Optimization (PPO) to learn the optimal UAV selection strategy. A Lagrangian optimization mechanism is incorporated to satisfy communication constraints such as delay, energy consumption, packet delivery ratio, and reliability. This integrated approach enables adaptive and efficient UAV selection, ensuring stable and reliable communication in highly dynamic vehicular environments.

Data Acquisition

The input data required for the proposed FL-PPO methodology is obtained from a simulated dynamic UAV-IoV environment. The dataset consists of both vehicle and UAV communication parameters, including vehicle position, speed, UAV location, signal strength, communication delay, packet delivery ratio (PDR), energy consumption, and communication range. These parameters represent the real-time network conditions used to evaluate UAV reliability and support intelligent UAV selection during the decision-making process.

Data pre-processing

Table 4 Acquired data and preprocessing operations

| Input variable | Model bound / unit | Preprocessing | Purpose |
| --- | --- | --- | --- |
| Vehicle coordinates | 0–2000 m | Divide by area size | Vehicle location |
| Relative UAV coordinates | −2000–2000 m | Clip and map to [0, 1] | Relative geometry |
| UAV altitude | 80–200 m | Used in 3-D distance calculation | Aerial geometry |
| Signal quality | 0.05–1.00 (nominal bound) | Already normalized | Link strength |
| Delay | 0–100 ms (capped model bound) | Threshold/fuzzy evaluation | Responsiveness |
| PDR | 50–100% (nominal bound) | Percentage form | Reliability |
| UAV energy | 0–50 units | Multiply by two and clip | Energy percentage |
| Lagrangian multipliers | 0–10 | Internal state; clip after update | Adaptive constraint importance; not part of the 34-dimensional PPO observation |

To ensure that measurements expressed in different units and ranges contribute consistently to the learning agent, pre-processing is applied before the state vector is constructed. Let L = 2000 m denote the side length of the operational region. The normalized vehicle coordinates are computed as in Equation (12), relative UAV coordinates are clipped and mapped to [0, 1] through Equations (13) and (14), and Equation (15) converts internal UAV energy into a percentage-compatible value.

xv(t)=xv(t)L, yv(t)=yv(t)L	(12)

xu,v(t)=12[clip (xu(t)-xv(t)L,-1,1)+1]	(13)

yu,v(t)=12[clip (yu(t)-yv(t)L,-1,1)+1]	(14)

Eu%(t)=min (100,max (0,2Eu(t)))	(15)

The processed FL-PPO observation is given by Equation (16). It contains two normalized vehicle features and four features for each of eight UAVs, producing a 34-dimensional observation.

st=[xv,yv,{xu,v,yu,v,Sv,u,Eu50}u=18]	(16)

System model

A UAV-assisted IoV communication network is considered over a square operational region having dimensions 2000 × 2000 m². The network contains multiple moving vehicles together with eight candidate UAVs that provide aerial communication support whenever terrestrial communication becomes unreliable or unavailable. The candidate UAV set is defined by Equation (1), where U denotes the set of available UAVs participating in the communication process.

U={U1,U2,…,U8}	(1)

At every communication decision instant, one representative active vehicle must be associated with one UAV from the candidate fleet. The objective is to maximize communication reliability while simultaneously reducing delay, improving packet delivery ratio, preserving UAV energy, and avoiding poor-quality communication links. Figure 3 illustrates the communication topology considered in the proposed framework — vehicles move inside the UAV-assisted IoV service region while observing multiple candidate aerial communication links. Dashed connections represent feasible vehicle-to-UAV (V2U) links, whereas the highlighted connection denotes the UAV selected by the proposed Fuzzy LP-PPO policy; after each communication decision, the measured QoS parameters are returned to the controller and used to update the learning policy.

Figure 2 : UAV-IoV topology and communication-selection process of the proposed Fuzzy LP-PPO framework.

Let p_v(t) = [x_v(t), y_v(t)] represent the two-dimensional position of vehicle v at decision instant t, and p_u(t) = [x_u(t), y_u(t), z_u(t)] represent the three-dimensional position of UAV u. Initially, the horizontal positions of vehicles and UAVs are generated randomly inside the operational region. UAV altitude is initialized between 80 m and 200 m, while every UAV starts with a residual energy value randomly selected between 30 and 50 internal energy units. Throughout one training episode, UAV positions remain fixed, whereas vehicle locations change according to the mobility model described below; at the beginning of every new episode, all positions and UAV energy levels are regenerated.

The communication-selection problem is formulated as a Markov Decision Process (MDP). At each decision step t, the PPO agent observes the current state s_t, executes an action a_t, receives an immediate reward r_t, and then transitions to the next state s_(t+1). The observation vector contains normalized vehicle coordinates, relative UAV coordinates, signal-quality measurements, and normalized residual UAV energy, as given in Equation (2), producing a 34-dimensional observation consisting of two vehicle features and four communication features for each of the eight candidate UAVs. The action space of the proposed PPO agent is discrete, expressed as Equation (3), where every action corresponds to one position within the ranked list of candidate UAVs.

st=[xv,yv,{xu,v,yu,v,Sv,u,Eu}u=18]	(2)

A={0,1,2,3,4,5,6,7}	(3)

Figure 3 : Proposed Candidate UAV Screening and Ranked Action Mapping Architecture

The above Figure illustrates the overall workflow of the proposed FL-PPO framework for reliable UAV selection in dynamic UAV-IoV systems. Initially, vehicle, UAV, and communication state information are collected and used to compute important communication parameters such as distance, delay, signal strength, packet delivery ratio, and energy. These parameters are then pre-processed and evaluated using a fuzzy membership mechanism to estimate the reliability of each candidate UAV. Based on the reliability assessment, infeasible UAVs are rejected, while feasible candidates are forwarded for candidate scoring and ranking. Finally, the ranked action mapping module generates the action set that is provided to the FL-PPO agent for optimal UAV selection and reliable communication.

Vehicle mobility model

The proposed communication-selection framework operates in dynamic IoV environments where vehicle positions continuously change throughout the communication process; consequently, communication distance, signal quality, PDR, and transmission delay also vary with time. Unlike trajectory-planning approaches that optimize complete vehicle routes, the proposed framework focuses on communication-aware UAV selection, so only vehicle positions are updated at every communication decision step while UAV positions remain fixed throughout one training episode — reflecting practical emergency-communication and temporary-UAV-deployment scenarios in which aerial platforms hover over predefined service regions.

At the beginning of every episode, vehicle locations are initialized randomly inside the operational area. During each communication decision step, every vehicle performs a random movement whose magnitude depends on the current operating scenario. The vehicle position at the next communication instant is computed as in Equation (4), where Δp_v(t) denotes the random vehicle displacement during the current decision step, L represents the length of the simulation region, and clip(·) confines the updated position within the simulation boundaries. The displacement vector is generated independently for both horizontal coordinates at every decision instant, so the agent continuously observes changing communication conditions throughout one training episode.

pv(t+1)=clip (pv(t)+Δpv(t),0,L)	(4)

Urban-canyon scenario: The urban-canyon scenario represents densely populated metropolitan road networks containing numerous buildings and frequent communication blockages. Approximately 20 moving vehicles are deployed within the simulation region; vehicle motion is relatively slow because of traffic congestion, intersections, and frequent stopping events, so communication links change gradually, although severe signal degradation may occur because of obstruction and multipath propagation. This scenario evaluates the capability of the proposed framework to maintain reliable communication in dense urban environments.

Suburban-crossroads scenario: The suburban-crossroads scenario models moderately populated road networks with lower vehicle density and fewer communication obstacles. Approximately 15 vehicles move throughout the operational region with moderate velocities, and communication quality generally remains more stable than in urban environments because line-of-sight conditions occur more frequently. This scenario primarily evaluates communication efficiency together with UAV energy conservation under moderate traffic conditions.

Emergency-response scenario: The emergency-response scenario represents disaster-relief or emergency communication situations in which communication demands vary rapidly and vehicle movement becomes less predictable. Approximately 15 vehicles are deployed inside the operational area, and their movement is generated using larger random displacements than in the previous scenarios, producing a rapidly changing communication topology that evaluates the adaptability of the proposed Fuzzy LP-PPO framework under highly dynamic operating conditions.

Figure 4 : Intelligent Network Model of the Proposed FLP-PPO Framework

This figure presents the intelligent network architecture of the proposed Fuzzy Lagrangian Proximal Policy Optimization (FLP-PPO) framework for reliable UAV selection in dynamic UAV-IoV environments. The communication network comprises three representative vehicular scenarios, namely Urban Canyon, Suburban Crossroads, and Emergency Response, where vehicles continuously exchange information with multiple UAVs through Vehicle-to-UAV (V2U) communication links. The collected communication information, including communication distance, transmission delay, signal strength, packet delivery ratio (PDR), and residual UAV energy, is forwarded to the Edge Intelligence Node for local decision making. The QoS Monitoring module continuously evaluates these communication parameters, while the Hard Feasibility Mask eliminates UAVs that violate predefined QoS constraints. The remaining feasible UAVs are prioritized by the Fuzzy Priority Engine, which performs multi-criteria decision making under uncertain network conditions, and the Candidate Ranking module sgenerates an ordered list of reliable UAVs. The ranked candidates are then supplied to the Constraint-Aware PPO Core, where the Actor-Critic networks and the Adaptive Lagrangian Constraint Bank jointly optimize the UAV selection policy while satisfying multiple QoS requirements. Communication experiences are stored in the Experience Memory and periodically sampled to improve learning stability through mini-batch optimization. Finally, the optimized policy is synchronized by the Global Policy Controller and distributed to all UAVs, enabling adaptive, scalable, energy-efficient, and QoS-aware communication through the selection of the optimal UAV for reliable vehicular connectivity.

Communication and energy model

The communication model converts the positions of vehicles and UAVs into the communication measurements required for candidate selection, fuzzy inference, reward computation, and adaptive Lagrangian optimization. During every communication decision step, the environment evaluates communication distance, signal quality, communication delay, PDR, and residual UAV energy for each candidate UAV; these metrics collectively describe the instantaneous network state and serve as inputs to the proposed Fuzzy LP-PPO framework.

The three-dimensional Euclidean distance between vehicle v and UAV u is calculated as in Equation (5), where x_v(t) and y_v(t) denote the horizontal coordinates of vehicle v, x_u(t) and y_u(t) represent the horizontal coordinates of UAV u, and z_u(t) denotes the UAV altitude. The maximum nominal communication range is fixed at R_comm = 500 m, and communication signal quality decreases linearly with increasing distance while maintaining a small constant value for UAVs located outside the communication range, as expressed in Equation (6).

dv,u(t)=(xv(t)-xu(t))2+(yv(t)-yu(t))2+zu2(t)	(5)

Sv,u(t)={max (0.1,1-dv,u(t)Rcomm),dv,u(t)≤Rcomm,0.05,dv,u(t)>Rcomm.	(6)

Communication delay is modeled as a function of communication distance together with a small stochastic variation representing short-term channel fluctuations, as given in Equation (7), where D_v,u(t) denotes the communication delay in milliseconds and ξ_t is a uniformly distributed random variable introducing minor delay variations between communication links. The delay is limited to a maximum value of 100 ms, preventing unrealistic communication latency while maintaining consistent QoS evaluation throughout the learning process.

Dv,u(t)=min (100,dv,u(t)10+ξt), ξt∈{1,2,…,9}	(7)

Packet delivery ratio (PDR) is estimated using both communication signal quality and communication delay, as defined in Equation (8), where P_v,u(t) is expressed as a percentage. Higher signal-quality values increase successful packet delivery, whereas larger communication delays reduce overall transmission reliability; a lower bound of 50% prevents negative or physically unrealistic packet-delivery values and ensures numerical stability during reinforcement-learning optimization.

Pv,u(t)=max (50,100Sv,u(t)-Dv,u(t)2)	(8)

UAV residual energy evolves according to Equation (9), where E_u(t) denotes the residual energy of UAV u at decision step t and e_u(t) represents the communication energy consumed during that decision, randomly selected from 1–3 units to introduce moderate variability in battery depletion; UAVs that are not selected during the current step retain their existing energy values. Since internal energy values are not directly comparable with other QoS metrics, residual UAV energy is converted into a normalized percentage representation, as given in Equation (10), before being used by the fuzzy inference system and adaptive constraint optimization module.

Eu(t+1)=max (0,Eu(t)-eu(t)), eu(t)∈{1,2,3}	(9)

Eu%(t)=min (100,max (0,2Eu(t)))	(10)

To encourage long-term communication reliability, four soft QoS constraints are monitored throughout reinforcement learning, corresponding to a maximum communication delay of 100 ms, a minimum PDR of 48%, a minimum normalized UAV energy of 8%, and a minimum communication signal quality of 0.05. The corresponding non-negative constraint-violation functions are defined in Equation (11); each violation function becomes zero whenever the corresponding communication requirement is satisfied, and otherwise produces a positive penalty proportional to the magnitude of the QoS violation. These values are forwarded to the adaptive Lagrangian optimization module, where the associated multipliers are updated during training, so that repeatedly violated communication constraints receive progressively larger penalties, encouraging the PPO agent to learn communication policies that simultaneously maximize reward and satisfy long-term QoS requirements.

gD=max(0,Dv,u-100), gP=max(0,48-Pv,u), gE=max(0,8-Eu%), gS=max(0,0.05-Sv,u)	(11)

At the completion of every communication decision, the selected UAV, communication measurements, PDR, signal quality, delay, remaining UAV energy, received reward, selected action, and updated constraint multipliers are stored within the environment information record, supporting subsequent communication-performance analysis, convergence evaluation, energy-consumption assessment, and comparison with existing UAV-selection methods.

Table 5 Main system and training parameters

| Parameter | Value |
| --- | --- |
| Operational area | 2000 × 2000 m² |
| Number of candidate UAVs | 8 |
| UAV altitude range | 80–200 m |
| Communication range | 500 m |
| Initial UAV energy | 30–50 internal units |
| Energy drain per selected step | 1–3 units |
| Observation dimension | 34 |
| Action-space size | 8 discrete actions |
| Training scenarios | Urban canyon, suburban crossroads, emergency response |
| Default training episodes | 100 per scenario |
| Steps per episode | 50 |
| PPO learning rate | 3 × 10⁻⁴ |
| PPO batch size | 64 |
| PPO epochs per update | 10 |
| Discount factor (γ) | 0.99 |
| GAE parameter (λ_GAE) | 0.95 |
| PPO clipping parameter (ε) | 0.20 |
| Entropy coefficient | 0.02 |
| Actor network | Two hidden layers, 128 neurons each |
| Critic network | Two hidden layers, 128 neurons each |
| Lagrangian multiplier step size | 0.03 |
| Lagrangian multiplier range | [0, 10] |
| PPO rollout length (n_steps) | 200 |

The complete Fuzzy LP-PPO architecture proceeds as follows: the communication module calculates link measurements, the hard mask removes clearly infeasible UAVs, the fuzzy module assigns continuous priority, the ranking module orders candidates, and PPO selects a ranked action.

Table 6 Components of the proposed architecture

| Component | Input | Processing | Output |
| --- | --- | --- | --- |
| Scenario generator | Scenario ID and seed | Initialize mobility and UAVs | Environment state |
| Communication module | Vehicle/UAV coordinates | Compute distance, signal, delay, and PDR | Link measurements |
| Hard mask | Distance, delay, signal, and energy | Apply thresholds | Feasible set |
| Weighted clipped risk-priority module | Delay, PDR, signal, energy, and emergency indicator | Compute clipped linear risks and their weighted sum | Risk-aware priority F_v,u |
| Ranking module | Measurements and priority | Composite scoring | Ranked UAV list |
| PPO actor–critic | State s_t | Action probability/value | Action and V_φ(s_t) |
| Lagrangian module | Per-step delay, PDR, signal, and energy violations | Persist and update multipliers across episode boundaries | Training-wide adaptive penalty |
| Logging module | Action and metrics | Store records | Evaluation output |

Algorithm for fuzzy-priority estimation and candidate ranking

The fuzzy-priority module is the mechanism through which the proposed Fuzzy LP-PPO framework converts uncertain, gradually varying communication conditions into a single, actionable priority score — playing a role analogous to a composite risk index in other domains. Rather than assessing delay, PDR, signal quality, and residual energy independently through hard thresholds alone, the module estimates a continuous urgency value for every candidate vehicle–UAV link, allowing the ranking and reward mechanisms to distinguish between mildly degraded and severely degraded communication conditions.

Equations (17)–(20) define four normalized risk components for delay, PDR, signal quality, and energy, each mapped to the interval [0, 1] using a linear proximity measure between an acceptable operating value and a problematic threshold. Equation (21) combines the four components into the overall fuzzy priority F_v,u(t), with an additional additive boost applied whenever the scenario indicator I_emg signals emergency-response conditions, allowing such situations to receive greater attention during candidate ranking.

μD(t)=clip (Dv,u(t)-5545,0,1)	(17)

μP(t)=clip (55-Pv,u(t)7,0,1)	(18)

μS(t)=clip (0.18-Sv,u(t)0.13,0,1)	(19)

μE(t)=clip (35-Eu%(t)35,0,1)	(20)

Fv,u(t)=clip [0.35μD+0.25μP+0.25μS+0.15μE+0.10Iemg,0,1]	(21)

Table 7 Fuzzy-priority inputs

| Fuzzy input | Low risk | Increasing risk | Maximum risk |
| --- | --- | --- | --- |
| Delay | D ≤ 55 ms | 55–100 ms | D ≥ 100 ms |
| PDR | P ≥ 55% | 48–55% | P ≤ 48% |
| Signal | S ≥ 0.18 | 0.05–0.18 | S ≤ 0.05 |
| Energy | E% ≥ 35% | 0–35% | E% = 0 |
| Emergency | Normal | — | Emergency |

Each UAV is checked using the hard-feasibility condition in Equation (22); if no UAV passes the conditions, the entire fleet is retained so that the action set never becomes empty. Feasible candidates are scored using the composite ranking function in Equation (23), which combines signal quality, PDR, delay, normalized energy, fuzzy priority, and a small distance penalty, and PPO selects a position within the resulting ranked list as defined in Equation (24). This design mirrors the labeling and weighted-scoring strategy commonly used to convert multiple raw indicators into a single, ordinally meaningful index for downstream decision-making, while remaining tailored to the communication-selection problem addressed in this work.

Mu(t)={1,dv,u≤500, Eu%≥20, Sv,u≥0.10, Dv,u≤95,0,otherwise,	(22)

Cv,u=240Sv,u+2Pv,u-1.3Dv,u+2.5Eu%+35Fv,u-0.02dv,u	(23)

at∈{0,1,…,7}	(24)

Algorithm 1: Fuzzy-Based Candidate Screening and Ranking

Algorithm 1. Fuzzy-based candidate screening and ranking Input: p_v(t), {p_u(t)}, {E_u(t)}, I_emg(t) Output: Ranked candidate list C* 1: C* ← ∅ 2: for each UAV u = 1, ..., 8 do 3: Compute distance, signal, delay, PDR and normalized energy 4: Compute μ_D, μ_P, μ_S and μ_E using Equations (17)–(20) 5: Compute F_v,u using Equation (21) 6: Evaluate feasibility M_u using Equation (22) 7: Compute C_v,u using Equation (23) and store candidate 8: end for 9: If at least one candidate is feasible, remove infeasible candidates 10: Sort candidates by score, energy, signal and lower delay 11: return C*

The algorithm performs a single traversal of all candidate UAVs to compute the communication metrics, fuzzy membership values, fuzzy priority, feasibility, and candidate scores, requiring O(N) time. Subsequently, infeasible UAVs are filtered in linear time, which also requires O(N). Since the sorting operation dominates the overall execution time, the total time complexity of the proposed fuzzy-based candidate screening and ranking algorithm is O(N logN), while the preceding computations remain linear.

Overall Time Complexity: O(N logN)

Reward and adaptive Lagrangian formulation

Although the fuzzy priority guides candidate ranking, the reward function used to train the PPO agent must remain scientifically grounded in the underlying communication objective rather than in the fuzzy score alone. The QoS score, task-service reward, adaptive constraint penalty, and final reward are defined in Equations (25)–(28); Equation (25) rewards high signal quality, high PDR, low delay, and high residual energy, Equation (26) rewards successful task service while penalizing distance and energy expenditure, and Equation (27) accumulates the adaptive Lagrangian penalty across the four monitored constraints. The final reward in Equation (28) combines the task-service reward, the QoS score, the fuzzy priority, and the adaptive constraint penalty, so that the agent is rewarded for both immediate communication quality and for avoiding constraint violations. The initial multipliers are given in Equation (29), and Equation (30) increases the multiplier associated with any observed violation through a dual-ascent update with step size η_λ = 0.03, so that repeated violations of a particular constraint become progressively more expensive during training while satisfied constraints do not accumulate additional penalty.

Qv,u(t)=190Sv,u(t)+1.5Pv,u(t)-1.2Dv,u(t)+2.2Eu%(t)	(25)

Rtask(t)=15Nt(t)-0.002dv,u(t)-0.08eu(t)-0.05	(26)

Φ(t)=λDgD(t)+λPgP(t)+λEgE(t)+λSgS(t)	(27)

rt=Rtask(t)+Qv,u(t)+20Fv,u(t)-Φ(t)	(28)

λ(0)=[0.35,0.20,0.60,0.35]	(29)

λi(t+1)=clip [λi(t)+ηλgi(t),0,10], ηλ=0.03	(30)

## Policy optimization with PPO

The proposed model leverages the state representation derived from vehicle and UAV measurements, together with the fuzzy priority and adaptive Lagrangian penalty, to learn a UAV-selection policy through actor–critic PPO training. The actor network outputs the policy π_θ(a_t | s_t) over the discrete action space, as in Equation (31), and the critic network estimates the state-value function V_φ(s_t), as in Equation (32). During training, the probability ratio between the current and previous policy is computed as in Equation (33), the temporal-difference residual is computed as in Equation (34), and the generalized advantage estimate (GAE) is computed as in Equation (35).

πθ(at∣st)	(31)

Vϕ(st)=E [∑k=0∞γkrt+k∣st]	(32)

ρt(θ)=πθ(at∣st)πθold(at∣st)	(33)

δt=rt+γVϕ(st+1)-Vϕ(st)	(34)

A^t=∑l=0∞(γλGAE)lδt+l	(35)

The clipped surrogate objective in Equation (36) limits the magnitude of each policy update by clipping the probability ratio to the interval [1 − ε, 1 + ε], preventing excessively large or destabilizing updates. The critic loss in Equation (37) minimizes the squared error between the estimated value and the empirical return, and the total loss in Equation (38) combines the negative clipped objective, the weighted critic loss, and an entropy bonus that discourages premature convergence to a deterministic policy.

Lclip(θ)=E [min (ρtA^t,clip(ρt,1-ε,1+ε)A^t)]	(36)

LV(ϕ)=E [(Vϕ(st)-R^t)2]	(37)

L(θ,ϕ)=-Lclip(θ)+cVLV(ϕ)-cHH[πθ(·∣st)]	(38)

Before ordinary PPO learning begins, a fuzzy-guided warm start reduces unstructured initial exploration: 800 states are sampled and the fuzzy-ranked best candidate is stored as action zero, and six behavior-cloning epochs minimize the warm-start loss in Equation (39). This warm start allows the agent to begin training with informative UAV-selection behavior rather than fully random exploration, which is particularly beneficial given the discrete, ranked action space and the presence of multiple competing QoS objectives.

LWS(θ)=-1NWS∑logπθ(atF∣st), atF=0	(39

Algorithm 2: Adaptive Fuzzy LP-PPO Training

Algorithm 2. Adaptive Fuzzy LP-PPO training Input: scenarios Ω, episodes N_ep, horizon H, rollout length K=200, actor θ, critic φ, η_λ Output: trained policies {π_θ*} 1: for each scenario ω ∈ Ω do 2: Initialize actor, critic and persistent λ = [0.35, 0.20, 0.60, 0.35] 3: Perform fuzzy-guided warm start using 800 samples and six epochs 4: Initialize the on-policy rollout buffer 5: for episode = 1, ..., N_ep do 6: Reset mobility, UAV positions and energy; retain λ 7: for t = 0, ..., H−1 do 8: Observe s_t and execute Algorithm 1 9: Sample a_t ~ π_θ(a_t ¦ s_t) and map it to the effective UAV 10: Execute communication and calculate r_t 11: Compute violations and update persistent λ using Equation (30) 12: Update energy, vehicle position and next state 13: Store the transition in the on-policy rollout buffer 14: if K=200 rollout steps have been collected then 15: Compute GAE and update actor and critic for 10 PPO epochs 16: Clear the rollout buffer 17: end if 18: end for 19: Record episode metrics 20: end for 21: Save the scenario-specific policy 22: end for

Algorithm 3: Online UAV Selection

Algorithm 3. Online UAV selection Input: trained policy π_θ* and current network state Output: selected UAV u* 1: Construct normalized observation s_t 2: Execute Algorithm 1 to obtain C* 3: Compute π_θ*(· ¦ s_t) 4: Select a_t = argmax_a π_θ*(a ¦ s_t) 5: Map a_t to effective UAV u* 6: Establish communication and record QoS information 7: return u*

Algorithm 2 describes the complete training process, including fuzzy warm start, Lagrangian adaptation, and PPO optimization across all three scenarios, while Algorithm 3 describes the lightweight online decision process used once a scenario-specific policy has been trained: the trained actor network is queried once per communication decision, and the fuzzy-ranked candidate list from Algorithm 1 continues to be used at inference time to guarantee that the effective UAV mapped from the PPO action always corresponds to a communication-feasible candidate whenever one exists.

Computational complexity

The overall complexity of the Fuzzy LP-PPO methodology is determined by three core components: communication and fuzzy-priority evaluation, candidate ranking, and PPO policy inference. Let N_U denote the number of candidate UAVs. Communication and fuzzy evaluation require O(N_U) operations per decision, since every candidate is evaluated independently, while ranking the candidates by composite score requires O(N_U log N_U) operations, as given in Equation (40). If W_π denotes the number of actor-network parameters, the total deployment-time complexity of one UAV-selection decision is given by Equation (41). This complexity reflects the trade-off between the low per-decision cost of fuzzy screening and ranking and the fixed cost of a single forward pass through the trained actor network, and remains lightweight enough for repeated, real-time UAV-selection decisions in dynamic IoV environments. Table 9 summarizes the principal implementation parameters used throughout training and evaluation.

O(NUlogNU)	(40)

O(NUlogNU+Wπ)	(41)

Table 8 Principal implementation parameters

| Parameter | Value |
| --- | --- |
| Candidate UAVs | 8 |
| Observation dimension | 34 |
| Actions | 8 |
| Actor/Critic | 128–128 neurons |
| PPO learning rate | 3 × 10⁻⁴ |
| γ / λ_GAE / ε | 0.99 / 0.95 / 0.20 |
| Entropy coefficient | 0.02 |
| Batch size / PPO epochs | 64 / 10 |
| Warm-start samples / epochs | 800 / 6 |
| Episodes / steps per scenario | 100 / 50 |
| Lagrangian step / range | 0.03 / [0, 10] |
| PPO rollout length (n_steps) | 200 |

# References

[1] C. Chen, B. Gu, B. Lyu, S. Gong, Z. Liu, and Y. Fang, “Traffic-Aware Asynchronous Trajectory Planning and Scheduling in UAV-Assisted Wireless Networks With Heterogeneous Traffic Demands,” IEEE Transactions on Mobile Computing, early access, 2026, doi: 10.1109/TMC.2026.3656507.

[2] Y. Wang, J. Wang, J. Chen, X. Hou, Z. Wang, and C. Jiang, “Drone-Aided Secure Task Offloading Optimization for Internet of Vehicles: Review, Challenges and Method,” IEEE Transactions on Network Science and Engineering, vol. 13, pp. 4596–4615, 2026, doi: 10.1109/TNSE.2025.3641579.

[3] A. Alagha, M. Kadadha, R. Mizouni, S. Singh, J. Bentahar, and H. Otrok, “UAV-Assisted Internet of Vehicles: A Framework Empowered by Reinforcement Learning and Blockchain,” Vehicular Communications, vol. 52, Art. no. 100874, 2025, doi: 10.1016/j.vehcom.2025.100874.

[4] S. Goudarzi, S. A. Soleymani, M. H. Anisi, A. Jindal, and P. Xiao, “Optimizing UAV-Assisted Vehicular Edge Computing With Age of Information: A SAC-Based Solution,” IEEE Internet of Things Journal, vol. 12, no. 5, pp. 4555–4569, 2025, doi: 10.1109/JIOT.2025.3529836.

[5] S. A. Zakaryia, M. Meaad, T. Nabil, and M. K. Hussein, “Task Offloading and Resource Allocation for Multi-UAV Asset Edge Computing With Multi-Agent Deep Reinforcement Learning,” Computing, vol. 107, Art. no. 126, 2025, doi: 10.1007/s00607-025-01472-5.

[6] Y. Wu, Y. Huang, Z. Wang, and C. Xu, “Joint Caching and Computation in UAV-Assisted Vehicle Networks via Multi-Agent Deep Reinforcement Learning,” Drones, vol. 9, no. 7, Art. no. 456, 2025, doi: 10.3390/drones9070456.

[7] S. Xu, Q. Liu, C. Gong, and X. Wen, “Energy-Efficient Multi-Agent Deep Reinforcement Learning Task Offloading and Resource Allocation for UAV Edge Computing,” Sensors, vol. 25, no. 11, Art. no. 3403, 2025, doi: 10.3390/s25113403.

[8] J. Su and Y. Liu, “Task Offloading Decision Making for IoV Based on Deep Reinforcement Learning,” Scientific Reports, vol. 15, Art. no. 38586, 2025, doi: 10.1038/s41598-025-22499-w.

[9] E. Mustafa, J. Shuja, F. Rehman, A. Namoun, M. Bilal, and K. Bilal, “Deep Reinforcement Learning and SQP-Driven Task Offloading Decisions in Vehicular Edge Computing Networks,” Computer Networks, vol. 262, Art. no. 111180, 2025, doi: 10.1016/j.comnet.2025.111180.

[10] J. Xue, L. Wang, Q. Yu, and P. Mao, “Multi-Agent Deep Reinforcement Learning-Based Partial Offloading and Resource Allocation in Vehicular Edge Computing Networks,” Computer Communications, vol. 234, Art. no. 108081, 2025, doi: 10.1016/j.comcom.2025.108081.

[11] L. Wang, W. Zhou, and L. Zhai, “Task Offloading Strategy of Vehicle Edge Computing Based on Reinforcement Learning,” Journal of Network and Computer Applications, vol. 239, Art. no. 104195, 2025, doi: 10.1016/j.jnca.2025.104195.

[12] P. Zhang, E. Wang, L. Tan, N. Kumar, J. Wang, and K. Liu, “Enhancing Task Offloading in Vehicular Networks: A Multi-Agent Cloud-Edge-Device Framework,” Vehicular Communications, vol. 53, Art. no. 100898, 2025, doi: 10.1016/j.vehcom.2025.100898.

[13] J. Bo and X. Zhao, “Vehicle Edge Computing Task Offloading Strategy Based on Multi-Agent Deep Reinforcement Learning,” Journal of Grid Computing, vol. 23, Art. no. 13, 2025, doi: 10.1007/s10723-025-09800-x.

[14] X. Li, J. Li, Y. Sun, A. Muthanna, A. Hawbani, and L. Zhao, “Cache-Assisted Task Offloading in Vehicular Edge Computing: A Spatio-Temporal Deep Reinforcement Learning Approach,” Computer Communications, vol. 246, Art. no. 108351, 2026, doi: 10.1016/j.comcom.2025.108351.

[15] M. Deng, W. Liang, B. Qiu, and X. Xie, “Federated Deep Reinforcement Learning-Based Task Offloading Strategy With Game Theory in Vehicular Edge Computing,” Physical Communication, vol. 72, Art. no. 102802, 2025, doi: 10.1016/j.phycom.2025.102802.

[16] O. D. Pantaleon, A. B. Rahman, and E. E. Tsiropoulou, “BRAVE: Benefit-Aware Data Offloading in UAV Edge Computing Using Multi-Agent Reinforcement Learning,” Simulation Modelling Practice and Theory, vol. 140, Art. no. 103091, 2025, doi: 10.1016/j.simpat.2025.103091.

[17] Y. Cui, H. Qiang, H. Li, and H. Zhao, “Multi-Agent Based Online Cooperative Computation Offloading and Migration Strategy for Vehicular Edge Computing,” IET Intelligent Transport Systems, vol. 19, no. 1, Art. no. e70083, 2025, doi: 10.1049/itr2.70083.

[18] M. Ye, Z. Zhou, L. Zhu, F. Huang, T. Li, D. Wang, Y. Jin, and Y. He, “Improving Transmission in Integrated Unmanned Aerial Vehicle–Intelligent Connected Vehicle Networks With Selfish Nodes Using Opportunistic Approaches,” Drones, vol. 9, no. 1, Art. no. 12, 2025, doi: 10.3390/drones9010012.

[19] L. Zhang, C. Tian, T. Liu, X. Li, S. Mumtaz, and W. U. Khan, “Multi-Objective Decomposition Evolutionary DRL for UAV-Assisted MEC in Internet of Vehicles,” IEEE Transactions on Vehicular Technology, vol. 75, no. 2, pp. 3133–3148, 2026, doi: 10.1109/TVT.2025.3602933.

[20] M. Yan, H. Guo, R. Xiong, C. A. Chan, A. F. Gygax, and E. Wong, “Deep Graph Reinforcement Learning-Enabled Computation Offloading for UAV-Assisted Vehicular Edge Computing Networks,” in Proc. IEEE Global Communications Conference (GLOBECOM), 2025, pp. 2132–2137, doi: 10.1109/GLOBECOM59602.2025.11432146.